Guide of Import Telechotrol Template

As follows ,We use Cubase SX 3 for example， Cubase SX and Cubase SX 2 are all conformed.

- In the menu of Cubase SX 3  select  Device>Device Setup,as the picture followed,

2．Click ，select‘Generic Remote’，

3．In the pop-up window of‘Generic Remote’，click‘Import’，In the other pop-up window，pitch on the template which we want to import，and click‘Open’button，as the picture followed,

4．In the window of‘Generic Remote’, ‘MIDI Input’select ‘UF MIDI IN’and‘MIDI Output’ select ‘UF MIDI OUT’，as the following picture, At last click‘OK’ button which is in the window of‘Generic Remote’，done.