UF Brain Help Document

Welcome to choose UF Brain software!

- The UF Brain software is used for connection of the 2nd generation CME UF MIDI keyboard, and to modify and preserve data and program. With this software, you could define the devices of the UF MIDI keyboard easily, and update the firmware.

- The UF Brain software includes versions for WINDOWS and MAC OSX.

Installation:

- To install the software, double click on the mouse, and follow the indications on screen, you will complete the UF Brain software installation.

Operation:

- Connect the UF to the computer through USB, and turn on the power of the UF MIDI keyboard. Double click the shortcut pattern of the UF Brain on the desktop by mouse, or click “Start/Program/CME/UF Brain”, then the software can be started up.

Set up:

- Click the “Device” button at the top of the software window, type both of the names of the input and output as “USB AUDIO DEVICE” or “CME UF”, then click “OK” to confirm.

Update the UF Firmware:

- The software is installed into “C:\Program Files\CME\UF Brain\” by default, you can find a file named “UF/bin” there. Please copy the .bin document you would like to send to UF into this file.

- Before updating, please quit from the software program firstly, turn off the UF, and hold the “F1” + “F3” buttons on the UF panel, turn on the UF power again. Now, “UPD” should be displayed on the UF screen.

- Open the UF Brain software again and select the input and output ports of the device. Click on the drop-down menu of document type at the top left of the software window, select “*.bin”, find out the bin document needed to be sent, double click the document by mouse, select “send” in the pop-up dialog box and confirm, then it will start sending the document.

- When the document has been sent over, click “OK” to complete the sending process.

- Close the UF Brain software and turn off the UF, hold the “OCTAVE-” + “OCTAVE+” buttons, turn on the UF power again for initialization, then the update process will be completed.

- Updating the UF firmware and initialization will delete all the previous settings of UF!

- Please DO NOT apply the specialized firmware document which is not downloaded from CME website (http://www.cme-pro.com)!

Assign the Devices:

- At the bottom of the UF Brain software window, you could find the patterns corresponded to the UF devices, they are all can be re-assigned data and then be sent to the UF.

- In the State area, select one of the three layers corresponded to the “LAYER” button on the UF panel firstly.

- Select the data type of the assigned devices as Controller, Note or Program.

- In the Data Select area, set the channel number and click one of the controller numbers in the controller list.

- Drag this controller number to the pattern of the device you wish to assign and release it, then the assignation for the device is completed.

- The color of different data type is different as well: Green for Controller, Red for Note, Blue for Program.

- If the current selected data can not be assigned to the device, the color of the data window of the device will be gray.

- Click the “Default” button at the top of the software window, all the devices will set to default value.

Save, Send, Recall the Panel Snapshot:

- Click the “Send” button at the top of the software window, you could save the current status of all the devices on the window as a specialized system informational document as “sys” panel snapshot. This document will be saved into “C:\Program Files\CME\UF Brain\UF\sys\” by default. Then, the “Send” dialog box will pop up automatically, this type of custom information can be sent to the UF by confirmation.

- If you need to send the saved document, please click the drop-down menu at the top left of the UF Brain software window, select “*.sys”, and find the sys document which you would like to send, double click it by mouse, select “send” in the pop-up dialog box, then confirm it to send the snapshot to the UF.

- If you need to recall the saved document, please click the drop-down menu at the top left of the UF Brain software window, select “*.sys”, and find the sys document which you would like to recall, double click it by mouse, select “load” in the pop-up dialog box, then confirm it to load it up as the current status of the software.

Receive and Preserve the UF Panel Snapshot:

- Click the “Receive” button at the top of the software window, and apply the “DUMP” function in UF, then the current status of UF will be sent to the software.

- Name the received document, and click to preserve it, then the document will be preserved into the “sys” content automatically.

Modify the Velocity Curve:

- Please enter adjust velocity curves mode for UFv2:
【SHIFT】->B4【Adjust Velocity Curves】->【ENTER】

- Click the V.Curve button at the top of the software window, the velocity curve editing window should be open.

- In the FileName area of the right side, there are 10 velocity curve preset files for UFv2, select one of them, and click the Local button below, then the selected file will be applied as the actual operation file.

- In the V.Curve Show area of the left side, several points can be dragged by mouse, in order to modify the parameters of the curve, and the parameters displayed on the right will be changed accordingly.

- Choose the Real-time option at the bottom of the software window, the sonic effect of the real-time performance of the UFv2 corresponded to the change of the curve can be heard.

- When the modification of a velocity curve is completed, click the Save button, the curve can be named and saved to the file list of the software. Click the Delete button, the curve can be deleted (You can't delete 10 velocity curve preset files).

- Select one of the files in the file list, click Send button, thus this curve can be saved to the UFv2.

- Click the New button, a new velocity curve can be created and edited.

- Click the Default button, the current velocity curve can be set to default.

- If you need the latest document about UF, please click the “Homepage” button at the top of the software window whenever you like, go to the CME website and find more information you need.

CME
2007