VX Brain Help

Welcome to choose VX Brain software!

- The VX Brain software is used for connection to the CME VX MIDI keyboard, to update the firmware and modify the velocity curve.

- The VX Brain software is designed for WINDOWS and MAC OSX.

Installation:

- To install the software, double click the set-up file by mouse, and follow the instructions on screen will guide you through the process of VX Brain software installation.

Operation:

- Connect the VX to the computer through USB cable, and turn on the power of the VX MIDI keyboard. Double click the icon of the VX Brain on the desktop, or click "Start/Program/CME/ VX Brain", then the software can be started up.

Settings of the Software:

- Click the "Device" button at the top of the software window, select the names of the input and output as "USB AUDIO DEVICE" or "CME VX", then click "OK" to confirm.

Update the VX Firmware:

- The software is installed in "C:\Program Files\CME\ VxBrain\" by default, you can find a folder named "VxBrain\bin" there. Please copy the .bin file which you would like to send to VX into this folder.

- Turn on the power of the VX, open the VX Brain software and select the input and output ports of the device. Click on the drop-down menu of file type in the VX Brain software window, select "*.bin", find out the bin file needed to be sent, double click it, select "send" in the pop-up dialog box and confirm, then it will start sending the file.

- When the file has been transfered, click "OK" to complete the sending process.

- Close the VX Brain software and turn off the VX, hold the "OCTAVE-" + "OCTAVE+" buttons, turn on the VX power again for initialization, then the update process will be completed.

- Updating the VX firmware and initialization will overwrite all the previous settings of VX!

- Please DO NOT use the firmware file other than downloaded from CME website (http://www.cme-pro.com) !

Modify the Velocity Curve:

- Click the V.Curve button at the top of the software window, the velocity curve editing window should be open.

- In the FileName area of the right side, there are 10 velocity curve preset files for VX, select one of them, and click the Load button below, then the selected file will be applied as the actual operation file.

- In the V.Curve Show area of the left side, several points can be dragged by mouse, in order to modify the parameters of the curve, and the parameters displayed on the right will be changed accordingly.

- Choose the Real-time option at the bottom of the software window, the sonic effect of the real-time performance of the VX corresponded to the change of the curve can be heard.

- When the modification of a velocity curve is completed, click the Save button, the curve can be named   and saved to the file list of the software. Click the Delete button, the curve can be deleted (You can't delete 10 velocity curve preset files).

- Choose the velocity curve of VX in the User 01-10. Select one of the files in the file list, click Send button, thus this curve can be saved to the VX.

- Click the New button, a new velocity curve can be created and edited.

- Click the Default button, the current velocity curve can be set to default.

Send MIDI File to the VX:

- Put the MIDI file, which needs to be sent to the VX, in the VXBrain\mid\ folder listed in the software installation menu.

- Click the drop-down menu of file type in the VX Brain software window and select "*.mid", find the midi file to be sent and double click on it by mouse, choose the storage location for song in the pop-up dialog and click send, then the file will be sent to the VX.

- The VX can only receive MIDI file of format 0.

- A single MIDI file can not be bigger than 64kb.

For more latest documentations for VX, please click the "Homepage" button at the top of the software window, go to the CME website and find out more information you need.

CME
2007